DYNAMIC ROUTING LAB (RIP)

Cisco Packet Tracer Network Configuration & Analysis

Source File: Dynamic Routing.pkt

RIP-based dynamic routing across four Cisco 2911 routers and three LANs

# 1. Introduction

This lab demonstrates dynamic routing using the Routing Information Protocol (RIP) in Cisco Packet Tracer. Four Cisco 2911 routers form a routed core and edge topology connecting three separate PC LANs. Unlike static routing, the routers use RIP to exchange route information automatically.

# 2. Objectives

Configure IPv4 addresses on all router interfaces.

Connect three end-device LANs through a four-router topology.

Enable RIP and advertise the required directly connected networks.

Understand how routers learn remote networks dynamically.

Verify RIP-learned routes and end-to-end connectivity.

# 3. Network Topology

The topology contains a triangle between Router0, Router1, and Router2, with Router4 connected to Router2 as an edge branch. PC0, PC1, and PC2 each reside on their own /24 LAN.

## Example logical paths

PC0 → Router0 → Router2 → Router1 → PC1
PC0 → Router0 → Router2 → Router4 → PC2

The core triangle provides multiple possible routed paths; the forwarding path depends on routing information learned through RIP.

# 4. Devices and Roles

| Device | Model | Role |
| --- | --- | --- |
| Router0 | Cisco 2911 | Core router; connects PC0 LAN to the routed core |
| Router1 | Cisco 2911 | Core router; connects PC1 LAN to Router0 and Router2 |
| Router2 | Cisco 2911 | Core hub; connects Router0, Router1, and Router4 |
| Router4 | Cisco 2911 | Edge router; connects PC2 LAN to Router2 |
| PC0 | PC-PT | 192.168.10.2/24, gateway 192.168.10.1 |
| PC1 | PC-PT | 192.168.20.2/24, gateway 192.168.20.1 |
| PC2 | PC-PT | 192.168.40.2/24, gateway 192.168.40.1 |
| Power Distribution Device0 | iot_pdu | Present but unconnected |

# 5. IP Addressing Plan

| Link / LAN | CIDR | Router A | Router B | Host | Purpose |
| --- | --- | --- | --- | --- | --- |
| R0 ↔ R2 | 10.0.0.0/8 | R0 Gi0/0 = 10.0.0.2 | R2 Gi0/0 = 10.0.0.1 | — | Core link |
| R0 ↔ R1 | 30.0.0.0/8 | R0 Gi0/2 = 30.0.0.1 | R1 Gi0/2 = 30.0.0.2 | — | Core link |
| R1 ↔ R2 | 20.0.0.0/8 | R1 Gi0/1 = 20.0.0.2 | R2 Gi0/1 = 20.0.0.1 | — | Core link |
| R2 ↔ R4 | 40.0.0.0/8 | R2 Gi0/2 = 40.0.0.1 | R4 Gi0/0 = 40.0.0.2 | — | Edge link |
| PC0 LAN | 192.168.10.0/24 | R0 Gi0/1 = 192.168.10.1 | — | PC0 = 192.168.10.2 | LAN |
| PC1 LAN | 192.168.20.0/24 | R1 Gi0/0 = 192.168.20.1 | — | PC1 = 192.168.20.2 | LAN |
| PC2 LAN | 192.168.40.0/24 | R4 Gi0/1 = 192.168.40.1 | — | PC2 = 192.168.40.2 | LAN |

# 6. Routing Design with RIP

RIP is enabled on all four routers. Each router advertises the networks directly connected to it, allowing neighboring routers to learn remote routes dynamically.

## RIP network statements

| Router | Advertised Networks |
| --- | --- |
| Router0 | 10.0.0.0, 30.0.0.0, 192.168.10.0 |
| Router1 | 20.0.0.0, 30.0.0.0, 192.168.20.0 |
| Router2 | 10.0.0.0, 20.0.0.0, 40.0.0.0 |
| Router4 | 40.0.0.0, 192.168.40.0 |

## How dynamic learning works

Routers start with directly connected networks.

RIP shares routing information with neighboring routers.

Remote networks are learned dynamically.

The routing table is updated as route information is exchanged.

Remote PC subnets become reachable without static routes.

# 7. Router Configurations

## Router0

hostname Router0
interface GigabitEthernet0/0
 ip address 10.0.0.2 255.0.0.0
interface GigabitEthernet0/1
 ip address 192.168.10.1 255.255.255.0
interface GigabitEthernet0/2
 ip address 30.0.0.1 255.0.0.0
router rip
 network 10.0.0.0
 network 30.0.0.0
 network 192.168.10.0

## Router1

hostname Router1
interface GigabitEthernet0/0
 ip address 192.168.20.1 255.255.255.0
interface GigabitEthernet0/1
 ip address 20.0.0.2 255.0.0.0
interface GigabitEthernet0/2
 ip address 30.0.0.2 255.0.0.0
router rip
 network 20.0.0.0
 network 30.0.0.0
 network 192.168.20.0

## Router2

hostname Router2
interface GigabitEthernet0/0
 ip address 10.0.0.1 255.0.0.0
interface GigabitEthernet0/1
 ip address 20.0.0.1 255.0.0.0
interface GigabitEthernet0/2
 ip address 40.0.0.1 255.0.0.0
router rip
 network 10.0.0.0
 network 20.0.0.0
 network 40.0.0.0

## Router4

hostname Router4
interface GigabitEthernet0/0
 ip address 40.0.0.2 255.0.0.0
interface GigabitEthernet0/1
 ip address 192.168.40.1 255.255.255.0
interface GigabitEthernet0/2
 no ip address
 shutdown
router rip
 network 40.0.0.0
 network 192.168.40.0

# 8. PC Configuration

| PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 192.168.20.2 | 255.255.255.0 | 192.168.20.1 |
| PC2 | 192.168.40.2 | 255.255.255.0 | 192.168.40.1 |

# 9. Example Packet Flow

Consider PC0 (192.168.10.2) communicating with PC2 (192.168.40.2). PC0 sends the packet to Router0 at 192.168.10.1, and Router0 uses RIP-learned information to reach 192.168.40.0/24.

| Step | Device / Segment | Action |
| --- | --- | --- |
| 1 | PC0 | Sends the packet to Router0 at 192.168.10.1. |
| 2 | Router0 | Uses a RIP-learned route toward 192.168.40.0/24. |
| 3 | Core routers | Forward the packet using the RIP-selected path. |
| 4 | Router4 | Recognizes 192.168.40.0/24 as directly connected on Gi0/1. |
| 5 | PC2 | Receives the packet at 192.168.40.2. |

PC0 → Router0 → [RIP-selected core path] → Router4 → PC2

# 10. RIP vs Static Routing

| Aspect | Static Routing | RIP Dynamic Routing |
| --- | --- | --- |
| Route configuration | Manual | Learned through RIP updates |
| Route changes | Manual changes required | Can be learned dynamically |
| Scalability | More manual configuration | Reduces manual route configuration |
| Protocol overhead | No routing-protocol updates | Routing updates are exchanged |
| This lab | Not used | Primary routing mechanism |

# 11. Verification and Testing

Useful Cisco IOS commands:

show ip interface brief
show ip route
show ip protocols
show ip route rip
show running-config
ping <destination-ip>
traceroute <destination-ip>

Example end-to-end tests:

From PC0:
ping 192.168.40.2

From PC1:
ping 192.168.10.2
ping 192.168.40.2

From PC2:
ping 192.168.10.2
ping 192.168.20.2

RIP-learned entries are normally identified with code 'R' in the routing table, while directly connected networks appear as connected routes. The exact preferred path depends on the routing information learned during the simulation.

# 12. Key Networking Concepts

Dynamic routing protocols

Routing Information Protocol (RIP)

Directly connected and RIP-learned routes

Default gateways

Routing tables

End-to-end ICMP testing

# 13. Conclusion

This lab demonstrates dynamic routing with RIP across four Cisco 2911 routers. Each router advertises its directly connected networks, and RIP distributes that information so remote LANs can be reached without static routes. The topology illustrates how routing tables are built from information exchanged between neighboring routers.

# 14. Configuration Summary

| Device | Key Interfaces | RIP Advertisements |
| --- | --- | --- |
| Router0 | Gi0/0 10.0.0.2; Gi0/1 192.168.10.1; Gi0/2 30.0.0.1 | 10.0.0.0; 30.0.0.0; 192.168.10.0 |
| Router1 | Gi0/0 192.168.20.1; Gi0/1 20.0.0.2; Gi0/2 30.0.0.2 | 20.0.0.0; 30.0.0.0; 192.168.20.0 |
| Router2 | Gi0/0 10.0.0.1; Gi0/1 20.0.0.1; Gi0/2 40.0.0.1 | 10.0.0.0; 20.0.0.0; 40.0.0.0 |
| Router4 | Gi0/0 40.0.0.2; Gi0/1 192.168.40.1 | 40.0.0.0; 192.168.40.0 |